# Vector Field Histogram (VFH) Algorithm

## 1. Index Range

i, j ∈ [0, Cₘₐˣ]

where Cₘₐˣ is the maximum cell index of the occupancy grid.

## 2. Distance Formula

dᵢ,ⱼ = √[(xᵢ − xᵣ)² + (yⱼ − yᵣ)²]

Euclidean distance from cell (i,j) to the robot position (xᵣ, yᵣ).

## 3. Bearing Angle

βᵢ,ⱼ = tan⁻¹ⁿ ( (yⱼ − yᵣ) / (xᵢ − xᵣ) )

Angle from the robot to cell (i,j) in the occupancy grid.

## 4. Obstacle Vector Magnitude

mᵢ,ⱼ = c²ᵢ,ⱼ · (a − b · dᵢ,ⱼ)

where cᵢ,ⱼ is the certainty value of cell (i,j); a and b are positive constants.

## 5. Histogram Sector Index

k = ⌊ βᵢ,ⱼ / α ⌋

where α is the angular resolution of the polar histogram (sector width in degrees).

## 6. Polar Obstacle Density (POD) Histogram

hₖ = Σ mᵢ,ⱼ    for (i,j) ∈ Sₖ

Sum of all obstacle magnitudes that map to sector k. Sₖ is the set of cells whose bearing falls in sector k.

## 7. Masked/Smoothed Histogram

h'ₖ = (1 / (2l+1)) · Σᵢ₌ₗ⁺ₗ hₖ₊ᵢ

Smoothed histogram value at sector k using a moving average window of size (2l+1).

## 8. Obstacle Threshold Function

Hᵇₖ = { 1   if h'ₖ > Tʰⁱʰʰ, 0   if h'ₖ < Tₗₒʷ, h'ₖ   otherwise }

Classifies each sector as blocked (1), free (0), or uncertain (h'ₖ) based on threshold values Tʰⁱʰʰ and Tₗₒʷ.

## 9. Steering Direction

θₛₜ⃑⃑ᵣ = θₜ      (target direction)

θₛₜ⃑⃑ᵣ = (θₗ⃑ₕₜ + θᵣᵢᵍʰₜ) / 2      (bisection of valley)

## 10. Control Commands

ω = kω · θₛₜ⃑⃑ᵣ

V = Vₘₐˣ · (1 − |θₛₜ⃑⃑ᵣ| / θₘₐˣ)

Angular velocity ω is proportional to steering angle. Linear velocity V decreases as steering angle increases.

# VFH Polar Histogram Description

The VFH polar histogram is a 1D representation of the obstacle density around the robot. The x-axis spans from −180° to +180°, representing all possible directions relative to the robot. The y-axis (P) represents the polar obstacle density (POD) — the sum of magnitude values from all obstacles in each angular sector.

High histogram bars indicate high obstacle density in that direction (blocked sectors). Low or zero bars indicate free space (candidate directions for navigation). The robot selects the steering direction within a valley (low-density region) that is closest to the target direction.

The histogram is updated every control cycle based on the current occupancy grid, enabling real-time obstacle avoidance in dynamic environments.